INTERNATIONAL

HYDROLOGICAL

PROGRAMME

BRESCE

UNESCO/BRESCE “TRANSKARSTBA” Programme

Internationally Shared Aquifers in the Balkan Region

## Transboundary Karst Aquifers

FINAL REPORT

## Annex 1: Data Compilation

##### June 2007

Prepared by

Unesco Chair and Network INWEB/Aristotle University of Thessaloniki, Greece

Annex 1 		Data  Compilation

ADDENDUM:   TRANSBOUNDARY GROUNDWATERS IN THE SEE REGION

| - INWEB no: 1 Aquifer: Secovlje-Dragonja/Dragonja |  |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Predominantly limestones, weak to medium links to surface waters - Groundwater flows from Slovenia to Croatia and Slovenia to Croatia - Part of the Istra system |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 21? |
|  | - Slovenia | - Croatia |  |  |
| - Area (km2) | - 20 | - 99 |  |  |
| - Water uses and functions | - Provides part of regional drinking water supply for the town of Piran | - Drinking water supply |  |  |
| - Pressure factors | - Tourism and transport | - Communities |  |  |
| - Problems related to groundwater quantity | - None | - None |  |  |
| - Problems related to groundwater quality | - Pollution from urbanisation and traffic | - Local, moderate pathogens |  |  |
| - Transboundary impacts | - None | - None |  |  |
| - Groundwater management measures | - Pumping station disconnected from water supply system | - Existing protection zones |  |  |
| - Status and what is most needed | - Delineation and enforcement of drinking water protection zones | - Agreed delineation of transboundary groundwater and monitoring |  |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification | - GWS ID 50811 | - HR 502 |  |  |
| - Notes |  | - Transboundary groundwater under consideration but not approved |  |  |

| - INWEB no: 2 Aquifer: ?/Mirna-Istra |  | - Shared by: Slovenia and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Cretaceous karstic limestones, weak to medium links to surface water systems, groundwater flow from Slovenia to Croatia - Part of the Istra system |  |  |  | - Mediterranean Sea basin |
|  |  |  |  | - Border length (km): 26? |
|  | - Slovenia |  | - Croatia |  |
| - Area (km2) | - ? |  | - 214 |  |
| - Water uses and functions | - ? Drinking water supply |  | - Drinking water supply |  |
| - Pressure factors | - Sparsely populated |  | - None? |  |
| - Problems related to groundwater quantity | - - |  | - None |  |
| - Problems related to groundwater quality | - - |  | - - |  |
| - Transboundary impacts | - - |  | - - |  |
| - Groundwater management measures | - - |  | - Existing protection zones |  |
| - Trends and future prospects | - - |  |  |  |
| - GWB | - Not identified |  | - HR 507, HR 516 |  |
| - Status and what is most needed |  |  |  |  |
| - Notes | - Not clear which groundwater systems in both countries correspond to each other; delineation of transboundary groundwaters by common research and bilateral expert group decision is needed |  | - Transboundary groundwater under consideration, but not approved |  |

| - INWEB no: 3 Aquifer: ?/Opatija |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- |
| - Type ? Predominantly limestones, groundwater flows from Slovenia to Croatia |  |  | - Mediterranean Sea Basin |
|  |  |  | - Border length (km): 21? |
|  | - Slovenia | - Croatia |  |
| - Area (km2) | - ? | - 302 |  |
| - Water uses and functions | - Drinking water | - Drinking water supply |  |
| - Pressure factors | - Main highway from Trieste to Rijeka – possibility of accidental spillage | - None? |  |
| - Problems related to groundwater quantity | - - | - None |  |
| - Problems related to groundwater quality | - - | - Local problems with salinity |  |
| - Transboundary impacts | - - | - - |  |
| - Groundwater management measures | - Protection zones of karst source Rizana | - Existing protection zones |  |
| - Status and what is most needed |  | - Agreed delineation of transboundary groundwater systems and monitoring |  |
| - GWB identification | - Not identified | - HR 515 |  |
| - Future trends and prospects | - - | - - |  |
| - Notes | - Not clear which groundwater systems in both countries correspond to each other; delineation of transboundary groundwaters by common research and bilateral expert group decision is needed | - Transboundary groundwater under consideration, but not approved |  |

| - INWEB no: 4 Aquifer: ?/Rijeka |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- |
| - Type? Predominantly limestones - Groundwater flows from Slovenia to Croatia - Part of the Istra system |  |  | - Mediterranean Sea basin |
|  |  |  | - Border length (km): 36? |
|  | - Slovenia | - Croatia |  |
| - Area (km2) | - ? | - 460 |  |
| - Water uses and functions | - Local drinking water supply | - Drinking water supply |  |
| - Pressure factors | - Main road from Ljubljana to Rijeka – possible accidental spills | - None |  |
| - Problems related to groundwater quantity | - - | - None |  |
| - Problems related to groundwater quality | - - | - None |  |
| - Transboundary impacts | - - | - None |  |
| - Groundwater management measures |  | - Existing protection zones of the city of Rijeka |  |
| - Status and what is most needed |  |  |  |
| - GWB identification |  | - HR 517, HR 518 |  |
| - Future trends and prospects |  |  |  |
| - Notes | - Not clear which groundwater systems in both countries correspond to each other; delineation of transboundary groundwaters by common research and bilateral expert group decision is needed | - Transboundary groundwater under consideration, but not approved |  |

| - INWEB no: 5 Aquifer: Cerknica/Kupa |  | - Shared by: Slovenia and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Triassic and Cretaceous limestones and dolomites, weak to medium links to surface water systems, groundwater flow from Croatia to Slovenia and Slovenia to Croatia |  |  |  | - Black Sea basin |
|  |  |  |  | - Border length (km): 32 |
|  | - Slovenia |  | - Croatia |  |
| - Area (km2) | - 238 |  | - 137 |  |
| - Water uses and functions | - Local drinking water supply, first karst spring of the Ljubljanica River (a karstic river with 7 surface and 6 underground stretches) |  | - Drinking water supply |  |
| - Pressure factors | - None, sparsely populated, forested with some extensive agriculture |  | - None, very scattered population |  |
| - Problems related to groundwater quantity | - None |  | - None |  |
| - Problems related to groundwater quality | - None, good chemical status |  | - Occasional bacteriological pollution |  |
| - Transboundary impacts | - None for quantity or quality |  | - None |  |
| - Groundwater management measures | - None |  | - Existing protection zones |  |
| - Trends and prospects |  |  |  |  |
| - GWB identification | - GWS ID 11823 |  | - HR 343 and HR 344 |  |
| - Status and what is most needed | - Not t risk. - It is unclear which groundwater systems in the two countries correspond to each other; delineation of transboundary groundwater systems needs common research and bilateral decision to propose a transboundary groundwater, if appropriate |  | - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 6 Aquifer: Radovic-Metlika/Zumberak |  |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Triassic dolomites, weak to medium links with surface water systems, groundwater flow from Croatia to Slovenia |  |  |  | - Black Sea basin |
|  |  |  |  | - Border length (km): 12? |
|  | - Slovenia | - Croatia |  |  |
| - Area (km2) | - 27 | - 158 |  |  |
| - Water uses and functions | - Drinking water supply to the town of Metlika (captured source Metliski Obrh) | - Dominantly drinking water supply |  |  |
| - Pressure factors | - Agricultural activities | - None |  |  |
| - Problems related to groundwater quantity | - None | - None |  |  |
| - Problems related to groundwater quality | - Excessive pesticide content | - None |  |  |
| - Transboundary impacts | - None for quantity or quality | - None |  |  |
| - Groundwater management measures | - None | - Need to establish protection zones |  |  |
| - Trends and future prospects |  |  |  |  |
| - GWB identification | - GWS ID 22931 | - HR 265 |  |  |
| - Status and what is most needed | - It is unclear which groundwater systems in the two countries correspond to each other; delineation of transboundary groundwater systems needs common research and bilateral expert group decision to propose a transboundary groundwater, if appropriate | - Agreed delineation of transboundary groundwaters, and monitoring |  |  |
| - Notes |  | - Transboundary aquifer under consideration, but not approved |  |  |

| - INWEB no: 7 Aquifer: Bregana-Obrezje/Sava-Samobor |  |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Quaternary alluvial sands and gravels, 5-10 m thick, strong link to surface waters of the Sava River, groundwater flow from Slovenia to Croatia |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 7 |
|  | - Slovenia | - Croatia |  |  |
| - Area (km2) | - 4 | - 54 |  |  |
| - Water uses and functions | - Local drinking water supply | - Dominantly drinking water, and some industry |  |  |
| - Pressure factors | - Surface water hydro-electric power schemes and associated river regulation on the Sava, transport routes | - Agriculture, waste disposal from communities extraction of river gravel and river regulation |  |  |
| - Problems related to groundwater quantity | - None | - Changes in groundwater level detected |  |  |
| - Problems related to groundwater quality | - None, chemical status good | - Hydrocarbons - oils and occasionally nitrogen, iron and manganese |  |  |
| - Transboundary impacts | - None | - From hydropower plants and extraction of gravel |  |  |
| - Groundwater management measures | - None | - Existing protection zones |  |  |
| - Trends and future prospects |  |  |  |  |
| - GWB identification | - GWS ID 12417 | - HR 188and HR 189 |  |  |
| - Status and what is most needed | - It is unclear which groundwater systems in the two countries correspond to each other; delineation of transboundary groundwater systems needs common research and bilateral expert group decision to propose a transboundary groundwater, if appropriate | - Agreed delineation of transboundary groundwaters, and monitoring |  |  |
| - Notes | - Very small part in Slovenia | - Transboundary aquifer under consideration, but not approved - Provides approx 1 m3/s supply to Zagreb by bank filtration |  |  |

| - INWEB no: 8 Aquifer: Bizeljsko/Sutla |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- |
| - Type 5, Triassic dolomites, weak links to surface water systems, groundwater flow from Croatia to Slovenia |  |  | - Black Sea Basin |
|  |  |  | - Border length (km): 4? |
|  | - Slovenia | - Croatia |  |
| - Area (km2) | - 180 | - 12 |  |
| - Water uses and functions | - Drinking water | - Local drinking water supply |  |
| - Pressure factors | - None | - None |  |
| - Problems related to groundwater quantity | - None | - Local lowering of groundwater levels detected |  |
| - Problems related to groundwater quality | - None, good chemical status | - No data |  |
| - Transboundary impacts | - None | - Indications that water supply abstraction for Podčetrtek impacts on groundwater levels |  |
| - Groundwater management measures | - None | - None |  |
| - Future trends and prospects |  |  |  |
| - GWB identification | - GWS ID 12415 | - HR 073 and HR 078 |  |
| - Status and what is most needed | - It is unclear which groundwater systems in the two countries correspond to each other; delineation of transboundary groundwater systems needs common research and bilateral expert group decision to propose a transboundary groundwater, if appropriate | - Need for coordination between areas on both sides - agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes | - Area uncertain – possibly only part of the Bizeljsko groundwater system is relevant | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 9 Aquifer: Ormoz-Sredisce ob Dravi/Drava-Varazdin |  |  | - Shared by: Slovenia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Quaternary sands and gravels, average thickness 5-10 m, strong links to surface water systems groundwater flow from Slovenia to Croatia |  |  |  | - Black Sea basin |
|  |  |  |  | - Border length (km): 26? |
|  | - Slovenia | - Croatia |  |  |
| - Area (km2) | - 27 | - 768 |  |  |
| - Water uses and functions | - Drinking water supply | - Drinking water supply |  |  |
| - Pressure factors | - Agriculture, hydropower schemes, Drava River regulation | - Agriculture and population of local communities |  |  |
| - Problems related to groundwater quantity | - None | - None |  |  |
| - Problems related to groundwater quality | - None, good chemical status | - Nitrate concentrations above the drinking water standard in the first shallow aquifer, in the second, deeper aquifer, the water is of good quality |  |  |
| - Transboundary impacts | - None |  |  |  |
| - Groundwater management measures | - None |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification | - GWS ID 32716 | - HR 037 and HR 038 |  |  |
| - Status and what is most needed | - - | - Agreed delineation of transboundary groundwaters, and monitoring |  |  |
| - Notes |  | - Transboundary aquifer under consideration, but not approved |  |  |

| - INWEB no: 9a Aquifer: Dolinsko-Ravensko/ Mura? |  | - Shared by: Slovenia and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type? Quaternary alluvial sands and gravel, groundwater hydraulically corresponding to surface water systems of the Mura River and in strong connection; groundwater flow from Slovenia to Croatia and from Croatia to Slovenia? |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km):? |
|  | - Slovenia |  | - Croatia |  |
| - Area (km2) | - 449? |  | - - |  |
| - Water uses and functions | - Drinking water supply of town Murska Sobota, local water supply systems |  | - - |  |
| - Pressure factors | - Intensive agriculture; pan European transport corridor |  | - - |  |
| - Problems related to groundwater quantity | - Degradation of the Mura River due to river regulation and hydropower schemes |  | - - |  |
| - Problems related to groundwater quality | - Nitrate, pesticides |  | - - |  |
| - Transboundary impacts | - None |  | - - |  |
| - Groundwater management measures | - None |  | - - |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification | - GWS ID 42813 |  | - None |  |
| - Status and what is needed | - At risk - Delineation of transboundary groundwater systems needs common research and bilateral expert group decision to propose a transboundary groundwater, if appropriate |  | - - |  |
| - Notes: | - Probably only part of the Dolinsko-Ravensko groundwater system is relevant |  | - According to existing data, no transboundary groundwater is recognised |  |

| - INWEB no: 10 Aquifer: Mura |  | - Shared by: Hungary and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 3/4, Quaternary alluvial aquifer of sands, silts and gravels generally only 5-10 m thick but up to 30 m in Hungary and 150 m in Croatia, strong links to surface waters of the Mura River, groundwater flow towards the river - Groundwater provides 90% of total water supply in the Croatian part and >80% in Hungary |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 52 |
|  | - Hungary |  | - Croatia |  |
| - Area (km2) | - 300 (Total groundwater body is 1933 km2) |  | - -? |  |
| - Water uses and functions | - >75% drinking water, <25% each for industry, irrigation and livestock, maintaining baseflow and support of ecosystems |  | - - |  |
| - Pressure factors | - Agriculture and settlements (fertilisers, pesticides, sewage, traffic), groundwater abstraction |  |  |  |
| - Problems related to groundwater quantity | - Local and moderate (at settlements) increased pumping lifts, reduced yields and baseflow, degradation of ecosystems |  |  |  |
| - Problems related to groundwater quality | - Local but severe nitrate from agriculture, sewers and septic tanks at up to 200 mg/l, pesticides at up to 0.1 μg/l |  |  |  |
| - Transboundary impacts | - None |  |  |  |
| - Groundwater management measures | - Groundwater abstraction management used and effective, transboundary institutions, monitoring, public awareness, protection zones, treatment need improvement, vulnerability mapping, regional flow modelling, good agricultural practices and priorities for waste water treatment, integration with river basin management need to be introduced |  |  |  |
| - Status and what is most needed | - ? - Evaluation of the utilisable resource |  |  |  |
| - Future trends and prospects | - Exporting drinking water |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 11 Aquifer: Drava/Drava West |  | - Shared by: Hungary and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 3/4, Quaternary alluvial aquifer of sands and gravels, of average thickness 10 m and maximum 70 m, medium to strong links to surface waters, groundwater flow from Hungary to Croatia, but mainly towards the border river. |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 31 |
|  | - Hungary |  | - Croatia |  |
| - Area (km2) | - 262 |  | - 97 |  |
| - Water uses and functions | - >75% drinking water, <25% each for irrigation, industry and livestock |  | - Local drinking water supply |  |
| - Pressure factors | - Agriculture (fertilisers and pesticides), sewage from settlements, traffic, gravel extraction under water in open pits |  | - Extraction of sand and gravel under water in pits |  |
| - Problems related to groundwater quantity | - Local increases in pumping lifts, reduction of borehole yields and baseflow and degradation of ecosystems |  | - Changes in groundwater levels detected |  |
| - Problems related to groundwater quality | - Widespread but moderate nitrate at up to 200 mg/l from agriculture, sewers and septic tanks, pesticides at up to 0.1 μg/l |  | - No data |  |
| - Transboundary impacts | - None for quantity or quality |  | - None |  |
| - Groundwater management measures | - Groundwater abstraction management used and effective, transboundary institutions, monitoring, protection zones need improvement, vulnerability mapping, regional flow modelling, good agricultural practices and priorities for waste water treatments integration to the river basin management, protection of open pit areas need to introduced |  | - None |  |
| - Future trends and prospects | - Evaluation of the utilisable resource |  |  |  |
| - GWB identification |  |  | - HR 039 |  |
| - Status and what is most needed | - ? - Exporting drinking water |  | - Probably not at risk - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 12 Aquifer: Baranja/Drava East |  | - Shared by: Hungary and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 4, Pleistocene and Holocene fluvial sands and gravels average thickness of 50 – 100 m and up to 200 m, weak to medium links to surface water systems, groundwater flow from Hungary to Croatia - Groundwater provides 90% of total supply in the Croatian part and >80%in the Hungarian part |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 67 |
|  | - Hungary |  | - Croatia |  |
| - Area (km2) | - 607 |  | - 955 |  |
| - Water uses and functions | - >75% drinking water, >25% each for irrigation, industry and livestock, maintaining baseflow and spring flow |  | - Drinking water supply |  |
| - Pressure factors | - Agriculture (fertilisers and pesticides), sewers and septic tanks, traffic |  | - None |  |
| - Problems related to groundwater quantity | - Local and moderate increases in pumping lifts, reductions in borehole yields and baseflow |  | - None |  |
| - Problems related to groundwater quality | - Widespread but moderate nitrate at up to 200 mg/l, local and moderate pesticides at up to 0.1 g/l, widespread but moderate arsenic at up to 50 g/l |  | - Naturally-occurring iron at 1-4 mg/l and arsenic at up to 500 µg/l |  |
| - Transboundary impacts | - None for quantity or quality |  | - None |  |
| - Groundwater management measures | - Control of groundwater abstraction by regulation used and effective, transboundary institutions, water use efficiency, monitoring, public awareness, protection zones, effluent treatment and data exchange need improvement, vulnerability mapping, regional flow modelling, better agricultural practices, priorities for wastewater treatment, integration with river basin management and arsenic removal need to be applied |  | - Vulnerability mapping, then good agricultural practices and priorities for wastewater treatment, integration into river basin management, and need to establish protection zones |  |
| - Future trends and prospects | - Evaluation of the utilisable resource, status of groundwater quality |  |  |  |
| - GWB identification |  |  | - HR 042 and HR 043 |  |
| - Status and what is most needed | - ? - Joint monitoring (mainly quantitative) and joint modelling is needed |  | - ? - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 13 Aquifer: West Serbia/Backa/Sava? |  |  | - Shared by: Serbia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 3, Eopleistocene alluvial aquifer of mainly medium and coarse grained sands and some gravels, of average thickness 20 m and up to 45 m, medium links to surface water systems, groundwater flow from Serbia to Croatia - Groundwater is about 70% of total water use in the Serbian part, 90% in Croatia |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): |
|  | - Serbia | - Croatia |  |  |
| - Area (km2) | - 2672 | - ? |  |  |
| - Water uses and functions | - 50-75% drinking water, <25% each for irrigation, industry and livestock | - Dominantly drinking water |  |  |
| - Pressure factors | - Abstraction | - None |  |  |
| - Problems related to groundwater quantity | - Local increase in pumping lifts and reduction in borehole yields | - None |  |  |
| - Problems related to groundwater quality | - Widespread naturally-occurring arsenic at 10-80 µg/l. Local ammonium and pathogens from sanitation | - No data, but probably naturally-occurring iron at 1-4 mg/l and arsenic at up to 500 µg/l |  |  |
| - Transboundary impacts | - None for quantity or quality | - No |  |  |
| - Groundwater management measures | - Existing quantity and quality monitoring need to be improved, other management measures needed |  |  |  |
| - Status and what is most needed | - ? | - ? |  |  |
| - Notes |  |  |  |  |
| - Future trends and prospects |  |  |  |  |

| - INWEB no: Aquifer: Srem/West Srem/Dunav |  |  | - Shared by: Serbia and Croatia |  |
| --- | --- | --- | --- | --- |
| - Type 3, Sequence of Pontian, Paludine and Eopleistocene sands, gravely sands and gravels of the Danube valley, of average thickness 80-150 m and up to 250-400 m, upper, shallow unconfined part has medium to strong links to surface water system, deeper parts confined or semi-confined by silts and clays, groundwater flow from Serbia to Croatia and also parallel to the river in a S and SW direction - Groundwater provides about 70% of total supply in the Serbian part and 90% in Croatia |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): |
|  | - Serbia | - Croatia |  |  |
| - Area (km2) | - 627 / >3500 | - ? |  |  |
| - Water uses and functions | - 50-75% drinking water, <25% each for irrigation, industry and livestock | - >75% drinking water |  |  |
| - Pressure factors | - Groundwater abstraction, agriculture, industry |  |  |  |
| - Problems related to groundwater quantity | - Local and severe increased pumping lifts and reduction of borehole yields |  |  |  |
| - Problems related to groundwater quality | - Local, moderate pesticides from irrigated agriculture, heavy metals, organics and hydrocarbons from industry | - Naturally-occurring Fe at 1-4 mg/l and arsenic at up to 500 µg/l |  |  |
| - Transboundary impacts | - None for quantity or quality | - No |  |  |
| - Groundwater management measures | - Existing quantity and quality monitoring need to be improved, as do abstraction control, protection zones and wastewater treatment, other management measures not yet used but needed |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Status and what is most needed | - ? |  |  |  |
| - Notes: | - Two responses from Serbia |  |  |  |

| - INWEB no: 14 Aquifer: Sava/Posavina I? |  | - Shared by: Croatia and Bosnia Herzegovina |  |  |
| --- | --- | --- | --- | --- |
| - Type 4, Quaternary alluvial sands, gravels, clays and marls averaging around 100 m thick in Croatia, less in BiH, weak to medium links to surface water systems, groundwater flow generally from south to north - Groundwater is 100% of total water use in the BiH part, 90% in the Croatian part |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 85 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - 250 |  | - 396 |  |
| - Water uses and functions | - Dominantly drinking water, smaller amounts (<25% each) for industry and livestock |  | - Drinking water supply |  |
| - Pressure factors | - Wastewater, industry and agriculture |  | - Agriculture |  |
| - Problems related to groundwater quantity | - None |  | - None |  |
| - Problems related to groundwater quality | - None |  | - Naturally-occurring iron at 1-4 mg/l and arsenic at up to 500 µg/l |  |
| - Transboundary impacts | - None |  | - No data |  |
| - Groundwater management measures | - Sava Commission. Abstraction management, quantity and quality monitoring, protection zones and agricultural measures are used but need improvement, water use efficiency and wastewater treatment are needed or planned |  | - Existing protection zones |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification | - TBGWB 14 ? |  | - HR 243 and HR 244 |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Notes | - In lower aquifer (depth 90 to 115 m), naturally-occurring iron is <0.7 mg/l |  |  |  |

| - INWEB no: 15 Aquifer: Kupa |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Triassic and Cretaceous karstic limestones and dolomites, strong links to surface water systems, groundwater flow from …… to ….. |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 130 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - ? |  | - 452 |  |
| - Water uses and functions | - No data |  | - Dominantly drinking water |  |
| - Pressure factors | - No data |  | - No data |  |
| - Problems related to groundwater quantity | - No data |  | - No data |  |
| - Problems related to groundwater quality | - No data |  | - No data |  |
| - Transboundary impacts | - N/A |  | - N/A |  |
| - Groundwater management measures | - _ |  | - Need to establish protection zones |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification |  |  | - HR 361 |  |
| - Status and what is most needed | - _ |  | - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes | - Possible transboundary aquifer should be considered |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 16 Aquifer: Una/Plesevica |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Thick Palaeolithic, Mesozoic and Cenozoic limestones and dolomites in hydraulic contact with overlying alluvial sediments, strong links with surface waters, flow from Croatia to BiH towards the Una River. |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 130 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - 108 |  | - 1592 |  |
| - Water uses and functions | - >75% to support ecosystems and fishing, 25-50% of abstraction is for drinking water supply |  | - Dominantly drinking water supply |  |
| - Pressure factors | - Solid waste disposal |  | - Sewage from communities |  |
| - Problems related to groundwater quantity | - Polluted water locally drawn into the aquifer |  | - None |  |
| - Problems related to groundwater quality | - Local but severe nitrogen, heavy metals and pathogens |  | - Nitrogen and pathogens |  |
| - Transboundary impacts | - Yes, for quality only |  | - Sinkholes in BiH with transboundary effects in Croatia |  |
| - Groundwater management measures | - Many used but need improving, others needed or currently planned |  | - Protection zones exist at Klokot, Privilica, Toplica, Ostrovica and need to be established Korenički Izvor, Stipinovac and Mlinac |  |
| - Future trends and prospects |  |  | - - |  |
| - GWB identification |  |  | - HR 359 and HR 360 |  |
| - Status and what is most needed | - ? |  | - Probably at risk - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved. |  |

| - INWEB no: 17 Aquifer: Krka |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Cretaceous karstic limestone, strong links to surface water system, groundwater flow from BiH to Croatia |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 42 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - 85 |  | - 414 |  |
| - Water uses and functions | - >95% to support ecosystems, <5% of abstraction is for drinking water supply |  | - Drinking water supply and hydroelectric power |  |
| - Pressure factors | - Solid waste disposal |  | - Population in communities and industry |  |
| - Problems related to groundwater quantity | - Reduced springflow and ecosystem degradation |  | - None |  |
| - Problems related to groundwater quality | - Polluted water locally drawn into the aquifer |  | - Occasionally, local and moderate pathogens |  |
| - Transboundary impacts | - No data (possibly for quality only) |  | - Sinkholes in BiH with transboundary effects in Croatia |  |
| - Groundwater management measures | - Quantity and quality monitoring need to be improved, as do abstraction control, protection zones and wastewater treatment |  | - Need to establish protection zones |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification |  |  | - HR 546, HR 547 and HR 548 |  |
| - Status and what is most needed | - Not at risk |  | - Not at risk but highly vulnerable - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 18 Aquifer: Cetina |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Palaeolithic, Mesozoic and Cenozoic karstic limestones of average thickness 500 m and maximum 1000 m, in hydraulic connection with recent sediments, groundwater flow from BiH to Croatia towards the Cetina River, strong links to surface water system |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 70 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - 2650 |  | - 587 |  |
| - Water uses and functions | - Up to 50% for HEP, smaller amounts for drinking water, irrigation, industry, mining and livestock |  | - Drinking water supply |  |
| - Pressure factors | - Solid waste disposal, wastewater, agriculture, industry |  | - None |  |
| - Problems related to groundwater quantity | - Widespread but moderate degradation of ecosystems, and polluted water drawn into the aquifer |  | - None |  |
| - Problems related to groundwater quality | - Local and moderate nitrogen, pesticides, heavy metals, pathogens, organics, hydrocarbons |  | - Local and moderate pathogens |  |
| - Transboundary impacts | - None for quantity or quality |  | - Sinkholes in BiH with transboundary effects in Croatia |  |
| - Groundwater management measures | - Quantity and quality monitoring need to be improved, as do abstraction control and protection zones |  | - Existing protection zones used, but needed at Vukovića Vrelo |  |
| - Future trends and prospects |  |  |  |  |
| - GWB identification |  |  | - HR 558 |  |
| - Status and what is most needed | - ? - eed to improve protection of upper catchment, vulnerability mapping planned, and improved wastewater treatment needed |  | - Not at risk - Agreed delineation of transboundary groundwaters, and monitoring |  |
| - Notes: |  |  | - Transboundary aquifer under consideration, but not approved - Includes the Glamočko-Kupreško and other Poljes with very large springs - Intensive agriculture in the coastal delta region |  |

| - INWEB no: 19 Aquifer: Neretva right |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Cretaceous and Neogene layered and massive limestones and dolomites, marls, clays, sandstones, breccias and conglomerates average thickness 250-600 m and up to 600-1000 m, strong link to surface waters, groundwater flow from BiH to Croatia |  |  |  | - Mediterranean Sea basin |
|  |  |  |  | - Border length (km): ? |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - >1600 |  | - 862 |  |
| - Water uses and functions | - Dominantly drinking water supply and HEP, some irrigation |  | - Drinking water supply |  |
| - Pressure factors | - Agriculture, sanitation, waste disposal and industry |  | - None |  |
| - Problems related to groundwater quantity | - Widespread but moderate drawing of polluted water into the aquifer, reduced springflow and ecosystem degradation |  | - None |  |
| - Problems related to groundwater quality | - Nitrogen, pathogens and organic compounds, widely but moderate |  | - Occasionally local and moderate pathogens –microbiological pollution |  |
| - Transboundary impacts | - Possibly for quality |  | - Improved connection with sink points in BiH and wells and springs in Croatia |  |
| - Groundwater management measures | - Groundwater quantity monitoring used but needs improvement, as do protection zones and wastewater treatment |  | - Existing protection zones for the Opačac and Prud spring systems |  |
| - Future trends and prospects |  |  | - Increased road construction and urbanisation in the Neretva delta, which needs protection of its wetlands, lakes and wildlife |  |
| - GWB identification |  |  | - HR 565, 566, 567, 569, 598, 573, 574 |  |
| - Status and what is most needed | - ? - Need to improve protection of upper catchment, vulnerability mapping planned |  | - Not at risk or probably not at risk - Agreed delineation of transboundary groundwaters monitoring are needed |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved |  |

| - INWEB no: 20 Aquifer: Trebisnjica/Neretva left |  | - Shared by: Bosnia Herzegovina and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Jurassic, Cretaceous and Eocene layered and massive limestones, marls, clays with coals, sandstones, breccias and conglomerates over 1000 m thick, groundwater flow from BiH to Croatia, weak to medium links to surface water systems |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 124 |
|  | - Bosnia Herzegovina |  | - Croatia |  |
| - Area (km2) | - >2000 |  | - 242 |  |
| - Water uses and functions | - HEP and drinking water supply and also to support ecosystems |  | - Dominantly drinking water supply – Slano and the Ombla spring |  |
| - Major pressure factors | - Agriculture, sanitation, waste disposal |  | - None |  |
| - Problems related to groundwater quantity | - Widespread but moderate drawing of polluted water into the aquifer, reduced springflow and ecosystem degradation |  | - None |  |
| - Problems related to groundwater quality | - Nitrogen and pathogens, widely but moderate |  | - Natural saline intrusion and occasionally microbiologic pollution |  |
| - Transboundary impacts | - Decline of groundwater levels and increased groundwater pollution |  | - Improved connection with sink points in BiH and wells and springs in Croatia |  |
| - Groundwater management measures | - Transboundary agreements and data exchange used, but need improvement, monitoring is needed |  | - Need to establish protection zones |  |
| - Trends and future prospects |  |  | - Increased development pressures on the Neretva delta |  |
| - GWB identification |  |  | - HR 576, 576a, 577, 578, 580, 581, 585, 586 |  |
| - Status and what is most needed | - ? - Need to improve protection of upper catchment, vulnerability mapping planned, and improved wastewater treatment needed. Evaluation of the utilisable resource |  | - Probably not at risk - Agreed delineation of transboundary groundwaters and monitoring are needed |  |
| - Notes |  |  | - Transboundary aquifer under consideration, but not approved - Supplies Dubrovnik |  |

| - INWEB no: 21 Aquifer: Bileko Lake |  | - Shared by: Bosnia Herzegovina and Montenegro |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Thick Triassic, Jurassic and Cretaceous limestones and dolomites, weakly linked to surface waters, groundwater flow from Montenegro to BiH - Groundwater provides 100% of total water usage |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 90 |
|  | - Bosnia Herzegovina |  | - Montenegro |  |
| - Area (km2) | - >1000 |  | - ? |  |
| - Water uses and functions | - >75% for HEP, small amounts for drinking water and irrigation |  |  |  |
| - Pressure factors | - None |  |  |  |
| - Problems related to groundwater quantity | - Local, moderate degradation of ecosystems |  |  |  |
| - Problems related to groundwater quality | - None mentioned |  |  |  |
| - Transboundary impacts | - None for quantity or quality |  |  |  |
| - Groundwater management measures | - Existing groundwater quality monitoring needs improvement, other measures need to be applied |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |
| - Status and what is most needed | - ? |  |  |  |

| - INWEB no: 22 Aquifer: Dinaric litoral (west coast) |  | - Shared by: Montenegro and Croatia |  |  |
| --- | --- | --- | --- | --- |
| - Type 2, Jurassic and Cretaceous karstic limestones, average thickness 500 m and maximum greater than 1000 m, weakly connected to surface water systems - Groundwater provides 100% of total water use |  |  |  | - Mediterranean Sea basin |
|  |  |  |  | - Border length (km): |
|  | - Montenegro |  | - Croatia |  |
| - Area (km2) | - 200 |  | - - |  |
| - Water uses and functions | - 25-50% each for drinking water supply and industry, <25% each for irrigation and livestock |  | - - |  |
| - Pressure factors | - Abstraction of groundwater |  | - - |  |
| - Problems related to groundwater quantity | - Widespread and severe saline intrusion at the coast |  | - - |  |
| - Problems related to groundwater quality | - High salinity from the above |  | - - |  |
| - Transboundary impacts | - None for quantity or quality |  | - - |  |
| - Groundwater management measures | - Existing control of abstraction, efficiency of water use, groundwater monitoring, public awareness, protection zones and agricultural practices need to be improved, other measures need to be introduced |  | - - |  |
| - Future trends and prospects |  |  | - - |  |
| - Status and what is most needed | - ? |  | - - |  |
| - Notes |  |  | - According to existing data, there is not a transboundary groundwater here |  |

| - INWEB no: 23 Aquifer: Dinaric east coast/Skadar/Shkodra lake |  |  | - Shared by: Albania and Montenegro |  |
| --- | --- | --- | --- | --- |
| - Type 2, Jurassic, Cretaceous and lesser Palaeogene massive and stratified limestones and dolomites, average thickness of 500 m and maximum 1000 m, strong links to surface water systems, groundwater flow in both directions - Groundwater is 100% of total water use in Montenegro, 80-90% in Albania |  |  |  | - Mediterranean drainage basin |
|  |  |  |  | - Border length (km): 35 (excluding the lake border) |
|  | - Montenegro | - Albania |  |  |
| - Area (km2) | - 200 | - About 450 |  |  |
| - Water uses and functions | - 25-50% for drinking water supply, <25% each for irrigation, industry and livestock | - 50-75% for irrigation, <25% for drinking water supply, industry and livestock |  |  |
| - Pressure factors | - Groundwater abstraction | - Industry, waste disposal, sanitation and sewer leakage |  |  |
| - Problems related to groundwater quantity | - Widespread and severe sea water intrusion at the coast | - Local and moderate degradation of ecosystems around Shkodra Lake |  |  |
| - Problems related to groundwater quality | - Widespread and severe increased salinity | - Local and moderate pathogens from waste disposal, sanitation and sewer leakage |  |  |
| - Transboundary impacts | - None for quality or quantity | - Shkodra Lake is moderately polluted mainly by industrial wastewater and less by sewage effluents |  |  |
| - Groundwater management measures | - Abstraction management, efficient water use, monitoring, protection zones and good agricultural practices used but need improving, wastewater treatment needed | - Detailed hydrogeological and groundwater vulnerability mapping, monitoring of groundwater quantity and quality (particularly the large karst springs and those used for public water supply), public awareness campaigns, delineation of protection zones and wastewater treatment are all needed. Investigation of the relationships between karst groundwater and groundwater of the alluvial deposits with Shkodra Lake |  |  |
| - Future prospects and trends |  | - To increase collaboration, to build transboundary institutions and to create joint programmes for protecting karst and alluvial groundwater, as well as protecting Shkodra Lake and the surrounding wetlands. Improvement of village water supply is needed (and irrigation too) |  |  |
| - Status and what is most needed | - ? | No significant risk at the moment, but the area around the Shkodra Lake is developing rapidly. Long term measures to protect surface and groundwater are needed |  |  |
| - Notes |  | The realization of large planned engineering projects in this area could deeply influence surface and groundwaters. |  |  |

| - INWEB no: 24 Aquifer: Beli Drim/Drini Bardhe |  | - Shared by: Serbia and Albania |  |
| --- | --- | --- | --- |
| - Type 3, Lower and Upper Cretaceous karstic and dolomitised limestone, Miocene to Quaternary multilayer sequence 100 to 200 m thick, medium to strong links to surface waters, groundwater flow from Serbia to Albania - Groundwater is 30 % of total water use in the Serbian part and 60-70% in the Albanian |  |  | - Mediterranean Sea Basin |
|  |  |  | - Border length (km): 30 |
|  | - Serbia | - Albania |  |
| - Area (km2) | - 1000 | - 170 |  |
| - Water uses and functions | - 25-50% for irrigation, <25% for drinking water and industry, maintain baseflow | - 75% for irrigation, <25% each for drinking water and livestock, maintain baseflow |  |
| - Pressure factors | - Abstraction of groundwater | - Waste disposal, sanitation, sewer leakage |  |
| - Problems related to groundwater quantity | - None | - No problems |  |
| - Problems related to groundwater quality | - Nitrogen, pesticides and pathogens | - Local and moderate pathogens |  |
| - Transboundary impacts | - None for quantity or quality | - None for quantity or quality |  |
| - Groundwater management measures | - Numerous management measures mentioned as needed | - Monitoring of groundwater quantity and quality (particularly the big karst springs and those used for public water supply), public awareness campaigns, delineation of protection zones and wastewater treatment are needed, together with detailed hydrogeological and vulnerability mapping |  |
| - Future trends and prospects |  | - Better evaluation of the quantity and quality of groundwater |  |
| - Status and what is most needed | - ? | Not at risk, the population is small and at the moment the industry is not developed |  |
| - Notes | - Water level decline of 0.3 m/yr reported |  |  |

| - INWEB no: 25 Aquifer: Metohija |  | - Shared by: Serbia and Montenegro |  |  |
| --- | --- | --- | --- | --- |
| - Type 4, Tertiary (Miocene) alluvial sediments, average thickness 100 m and maximum 200 m, weak links to surface water systems - Groundwater is 20% of total water use |  |  |  | - Basin……. |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Montenegro |  |
| - Area (km2) | - 4000 |  | - ? |  |
| - Water uses and functions | - 25-50% for irrigation, <25% each for drinking water, industry and livestock, maintaining baseflow and spring flow |  |  |  |
| - Pressure factors | - Agriculture and local small industries |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | - Pesticides and industrial organic compounds |  |  |  |
| - Transboundary impacts | - None for quantity or quality |  |  |  |
| - Groundwater management measures | - Several mentioned as needed |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 26 Aquifer: Lim |  | - Shared by: Serbia and Montenegro |  |  |
| --- | --- | --- | --- | --- |
| - Type 1, Triassic-Cretaceous karstic limestone with overlying Quaternary alluvium of average thickness 200 m and maximum 400 m, medium connection to surface water, groundwater flow relatively equally shared in both - Groundwater is 40% of total water use in the Serbian part |  |  |  | - Basin…. |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Montenegro |  |
| - Area (km2) | - 600-800 |  | - ? |  |
| - Water uses and functions | - 25-50% for drinking water, <25% each for irrigation, mining and thermal spas |  |  |  |
| - Pressure factors | - Waste disposal, mining and industry |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | - Local but severe nitrogen, heavy metals, pathogens, industrial organics and hydrocarbons |  |  |  |
| - Transboundary impacts | - None for quantity, yes for quality due to pollution from (or of) Lim River in the upper catchment |  |  |  |
| - Groundwater management measures | - Abstraction management and protection zones used but need to be improved, other measures needed |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 27 Aquifer: Tara Massif |  | - Shared by: Serbia and Bosnia Herzegovina |  |  |
| --- | --- | --- | --- | --- |
| - Type 3, Triassic and Jurassic karstified limestones of 250-300 m average thickness and maximum 600 m, strong links to surface water systems, groundwater flow from Serbia to Bosnia Herzegovina? - Groundwater is 10% of total water use |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 117 |
|  | - Serbia |  | - Bosnia Herzegovina |  |
| - Area (km2) | - 211 |  | - >100 |  |
| - Water uses and functions | - Drinking water and fish breeding |  | - Drinking water, mostly small amounts for supplying villages |  |
| - Pressure factors | - Sanitation and septic tank leakage |  | - Wastewater, mining activity |  |
| - Problems related to groundwater quantity | - Local and severe degradation of ecosystems, local but moderate drawing of polluted water into the aquifer |  | - Local moderate loading of polluted water into the aquifer |  |
| - Problems related to groundwater quality | - Pathogens |  | - Bacteriological contamination |  |
| - Transboundary impacts | - None for quantity or quality |  | - None |  |
| - Groundwater management measures | - Groundwater abstraction management and quantity monitoring need improvement, other management measures need to be introduced or are currently planned |  | - Protection zones needed for some significant but as yet unused karst springs |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? - Important to provide protection for National Park |  | - ? |  |
| - Notes |  |  | - Negligible conditions for nomination as a transboundary groundwater |  |

| - INWEB no: 28 Aquifer: Macva-Semberija |  | - Shared by: Serbia and Bosnia Herzegovina |  |  |
| --- | --- | --- | --- | --- |
| - Type 3/4, Lower Pleistocene alluvial sands, sandy gravels with clayey lenses of 35-60 m average thickness and maximum 75-100 m, overlying multiple aquifer sequence, including karstified Triassic limestones, strong links to surface water systems, dominant flow from the Drina River to the Sava - Groundwater is 40-60% of total water use in the Serbian part, and 100% in the Bosnia part |  |  |  | - Black Sea basin |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Bosnia Herzegovina |  |
| - Area (km2) | - 967 / >2000 |  | - >250 |  |
| - Water uses and functions | - 50-75% drinking water, <25% each for irrigation, industry and livestock, and support of ecosystems |  | - Drinking water, irrigation, industry and livestock |  |
| - Pressure factors | - Agriculture and sanitation, some industry |  | - Agriculture and sanitation |  |
| - Problems related to groundwater quantity | - Local and moderate increase in pumping lifts, no declines in groundwater levels |  | - Local and moderate increase in pumping lifts, no significant declines in groundwater levels |  |
| - Problems related to groundwater quality | - Local and moderate nitrogen and pesticides from agriculture, local and moderate heavy metals and organics from industry |  | - Local and moderate nitrogen and pesticides from agriculture |  |
| - Transboundary impacts | - None for quantity, minor for quality |  | - None |  |
| - Groundwater management measures | - Abstraction control, monitoring of groundwater, protection zones and wastewater treatment need improvement, other management measures need to be introduced or are currently planned |  | - Sava Commission, groundwater abstraction regulation and quantity monitoring, protection zones, and good agricultural practices used and effective, water use efficiency, public awareness, wastewater treatment need to be applied |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Notes |  |  |  |  |

| - INWEB no: 29 Aquifer: Northeast Backa/Danube-Tisza Interfluve |  |  | - Shared by: Serbia and Hungary |  |
| --- | --- | --- | --- | --- |
| - Type 5, Part of North Panonian basin, Miocene and Eopleistocene alluvial sediments, predominantly sands with clayey lenses of average thickness 50-100 m and maximum 125-150 m in Serbia, average 250 m and maximum 700 m in Hungary, medium to strong links to surface waters, groundwater flow from Hungary to Serbia - Groundwater is 80% of total use and provides 100% of drinking water supply in Serbia, >80% of total supply in the Hungarian part |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 169 |
|  | - Serbia | - Hungary |  |  |
| - Area (km2) | - 4020 | - 9545 |  |  |
| - Water uses and functions | - >75% drinking water, <25% each for irrigation, industry and livestock | - >75% drinking water, <25% each for irrigation, industry and livestock, support of ecosytems |  |  |
| - Pressure factors | - Abstraction of groundwater | - Abstraction, agriculture, sewers and septic tanks |  |  |
| - Problems related to groundwater quantity | - Local and severe increased pumping lifts and reduction in borehole yields, local and moderate land subsidence | - Local and moderate increased pumping lifts, reduced borehole yields and baseflow, and degradation of ecosystems |  |  |
| - Problems related to groundwater quality | - Widespread and severe naturally occurring arsenic at 10-50 µg/l, widespread nitrogen and pathogens from sanitation, organic compounds | - Widespread and severe naturally occurring arsenic at 10-200 µg/l, widespread but moderate nitrate at up to 200 mg/l, pesticides at up to 0.1 μg/l |  |  |
| - Transboundary impacts | - Insufficient information to know, or possibly for quantity | - None |  |  |
| - Groundwater management measures | - Abstraction management effective, water efficiency, existing monitoring, protection zones, agricultural practices need to be improved, other measures need to be introduced | - Groundwater abstraction regulation used and effective, water use efficiency, monitoring, public awareness, protection zones and wsastewater treatment needed, vulnerability mapping, regional flow modelling, good agricultural practices and priorities for wastewater treatment, integration with river basin management, arsenic treatment or import of arsenic free water are needed |  |  |
| - Future trends and prospects |  | - Evaluation of the utilisable resource |  |  |
| - Status and what is most needed | - ? - Local declines in groundwater level of 0.5 m/yr, and 0.1 my/r more widely, | - ? - Joint monitoring (mainly quantitative) and joint modelling is needed |  |  |
| - Notes | - groundwater abstraction in both countries exceeds recharge |  |  |  |

| - INWEB no: 30 Aquifer: Banat |  | - Shared by: Serbia and Romania |  |  |
| --- | --- | --- | --- | --- |
| - Type 4 or 5, Alluvial aquifer of sands and gravels of Tertiary to Pleistocene age, weak links to surface water systems, groundwater flow from Romania to Serbia - Groundwater is up to 90% of total water use |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 225 |
|  | - Romania |  | - Serbia |  |
| - Area (km2) | - 11408 |  | - 4231 |  |
| - Water uses and functions | - 50% drinking water, 30% for industry and 20% for irrigation |  | - >75% drinking water, >10% each for irrigation, industry, livestock and spa |  |
| - Pressure factors | - None mentioned |  | - Sanitation, irrigated agriculture, waste disposal, industry, oilfields |  |
| - Problems related to groundwater quantity | - Local and moderate increases in pumping lifts |  | - Local, severe increase in pumping lifts and decrease of borehole yields, and declining groundwater levels of 0.5 m/yr locally - Some degradation of ecosystems |  |
| - Problems related to groundwater quality | - None mentioned |  | - Local, moderate, nitrogen, pesticides & pathogens, more widespread heavy metals, and organic pollutants - Widespread high natural arsenic concentrations (10-80 g/l) |  |
| - Transboundary impacts | - Reported as none for quantity and quality |  | - Yes, declining groundwater levels and quality |  |
| - Groundwater management measures | - None reported as already in use, a wide range of measures are currently planned |  | - Monitoring of quantity and quality needs improvement, other measures need to be introduced or are planned |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 31 Aquifer: Miroc and Golubac |  | - Shared by: Serbia and Romania |  |  |
| --- | --- | --- | --- | --- |
| - Type 1, Jurassic and Cretaceous karstic limestones average thickness 100 m and maximum thickness 300 m, weak links to surface water systems, groundwater flow from the border into both countries - Groundwater is about 15% of total water use |  |  |  | - Basin |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Romania |  |
| - Area (km2) | - 300 |  | - ? |  |
| - Water uses and functions | - Drinking water |  |  |  |
| - Pressure factors | - Waste disposal and industry |  | - None? |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | - Local and moderate nitrogen, pathogens and organics from waste disposal, local but severe heavy metals from industry |  |  |  |
| - Transboundary impacts | - None for quantity, yes for quality |  |  |  |
| - Groundwater management measures | - Protection zones and treatment need improvement, other measures need to be introduced |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Notes |  |  |  |  |

| - INWEB no: 32 Aquifer: Dacian basin |  | - Shared by: Serbia and Romania |  |  |
| --- | --- | --- | --- | --- |
| - Type 4, Tertiary, Badenian-Pleistocene alluvial sediments, sands, conglomerates and gravels average thickness 300 m and maximum 2000 m, medium links to surface water systems, groundwater flow from Serbia to Romania - Groundwater is 25% of total water use |  |  |  | - Basin |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Romania |  |
| - Area (km2) | - 1000-2000 |  | - ? |  |
| - Water uses and functions | - <25% each for drinking water, irrigation and industry |  |  |  |
| - Pressure factors | - Waste disposal |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | - Local and moderate nitrate, pathogens, hydrocarbons, organics, salinity from waste disposal |  | - Naturally-occurring Fe at 1-4 mg/l and arsenic at up to 500 µg/l |  |
| - Transboundary impacts | - None for quantity or quality |  |  |  |
| - Groundwater management measures | - Protection zones need to be improved, other measures need to be introduced |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Notes |  |  |  |  |

| - INWEB no: 33 Aquifer: Timok |  | - Shared by: Serbia and Bulgaria |  |
| --- | --- | --- | --- |
| - Type 3, Quaternary alluvial sediments, sands, gravels and conglomerates of average thickness 5 m and maximum 20 m, strong links to surface water systems of the Timok River, groundwater flow equally shared - Groundwater is about 15% of total water use |  |  | - Basin |
|  |  |  | - Border length (km): 234? |
|  | - Serbia | - Bulgaria |  |
| - Area (km2) | - 200 | - ? |  |
| - Water uses and functions | - Drinking water and irrigation |  |  |
| - Pressure factors | - Mining and waste disposal |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |
| - Problems related to groundwater quality | - Widespread and severe heavy metals and industrial organic compounds from mining, local pathogens and organics from waste disposal |  |  |
| - Transboundary impacts | - None |  |  |
| - Groundwater management measures | - Monitoring, protection zones and waste water treatment need to be improved, other management methods need to be introduced |  |  |
| - Future trends and prospects |  |  |  |
| - Status and what is most needed | - ? | - ? |  |
| - Notes |  |  |  |

| - INWEB no: 34 Aquifer: Stara Planina/Salasha Montana |  |  | - Shared by: Serbia and Bulgaria |  |
| --- | --- | --- | --- | --- |
| - Type 2, Triassic and Cretaceous karstic limestones with some overlying Quaternary alluvium, average thickness 100 m and maximum 400 m, medium links to surface water systems, groundwater flow from Bulgaria to Serbia - Groundwater is about 50% of total water use |  |  |  | - Basin…….. |
|  |  |  |  | - Border length (km): |
|  | - Serbia | - Bulgaria |  |  |
| - Area (km2) | - 500 | - ? |  |  |
| - Water uses and functions | - 25-50% drinking water, <25% each for irrigation, industry, thermal spa and livestock |  |  |  |
| - Pressure factors | - Waste disposal and industry |  |  |  |
| - Problems related to groundwater quantity | - Local and moderate reduction in baseflow and degradation of ecosystems, with polluted water drawn into aquifer |  |  |  |
| - Problems related to groundwater quality | - Local and moderate nitrogen and pathogens from waste disposal, more severe heavy metals from industry and organic pollutants from waste disposal |  |  |  |
| - Transboundary impacts | - Not for quantity or quality |  |  |  |
| - Groundwater management measures | - Abstraction management, protection zones and treatment of industrial effluents need improvement, other measures need to be introduced or are currently planned |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Status and what is most needed |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 35 Aquifer: Nishava and Tran/Vidlic? |  | - Shared by: Serbia and Bulgaria |  |  |
| --- | --- | --- | --- | --- |
| - Type 5, Lower Cretaceous karstified dolomitic limestones average thickness 200 m and maximum 400 m, strong links to surface water systems, groundwater flow from…… - Proportion groundwater of total water use unknown |  |  |  | - Basin….. |
|  |  |  |  | - Border length (km): |
|  | - Serbia |  | - Bulgaria |  |
| - Area (km2) | - 285 |  | - ? |  |
| - Water uses and functions | - 50-75% drinking water, <25% each for industry and livestock, and support of ecosytems |  |  |  |
| - Pressure factors | - Agriculture |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | - Local but severe pathogens from farming |  |  |  |
| - Transboundary impacts | - None for quantity or quality |  |  |  |
| - Groundwater management measures | - Many management measures need to be applied |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Status and what is most needed | - ? |  | - ? |  |
| - Notes |  |  |  |  |

| - INWEB no: 36 Aquifer: Zemen |  |  | - Shared by: Serbia and Bulgaria |  |
| --- | --- | --- | --- | --- |
| - Type ?, Middle and Upper Triassic dolomites and dolomitic limestones on Paleozoic crystalline formations |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): ? |
|  | - Serbia | - Bulgaria |  |  |
| - Area (km2) | - >200? | - 180? |  |  |
| - Water uses and functions | - Only surface water from the confluence of the Strymen river |  |  |  |
| - Pressure factors | - Shortage of water |  |  |  |
| - Problems related to groundwater quantity | - Difficulty of meeting water supply demands |  |  |  |
| - Problems related to groundwater quality | - Local and moderate nitrogen and pathogens from waste disposal |  |  |  |
| - Transboundary impacts | - None for quantity or quality |  |  |  |
| - Groundwater management measures | - Abstraction management needed |  |  |  |
| - Status and what is most needed | - Bringing water supply from outside the aquifer |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes | - Catchment of the River Struma |  |  |  |

| - INWEB no: 37 Aquifer: FYROM-SW Serbia |  |  | - Shared by: FYROM and Serbia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Mesozoic and Paleozoic rocky masses (generally schists and limestones) with karstic aquifers, minor alluvial sediments, thickness of aquifers is variable, depending on the tectonic characteristics, groundwater flow occurs in both directions, but more from Serbia to FYROM |  |  |  | - Mediterraneann Sea Basin |
|  |  |  |  | - Border length (km): |
|  | - FYROM | - Serbia |  |  |
| - Area (km2) |  | - 300 |  |  |
| - Water uses and functions | - Water for drinking, little irrigation and mining | - 50% for drinking water supply, 50% for agriculture |  |  |
| - Pressure factors | - Local leakage of groundwater from wells and groundwater from springs | - Agriculture |  |  |
| - Problems related to groundwater quantity | - Local reduction of boreholes yields and spring discharges | - None |  |  |
| - Problems related to groundwater quality | - Local from agriculture | - Local? |  |  |
| - Transboundary impacts | - Only for quantity of groundwater | - None for quantity, local for quality |  |  |
| - Groundwater management measures | - Monitoring of quantity and quality, protection zones, hydrogeological mapping, good agricultural practices, exchange of data between countries are used, other measures need to be applied or are planned. | - Abstraction management used, other measures planned, such as monitoring |  |  |
| - Status and what is most needed |  | - Water supply only from …. |  |  |
| - Future trends and prospects | - ? |  |  |  |
| - Notes | - After definition of the border between the two counties, all data must be defined and data must be exchanged |  |  |  |

| - INWEB no: 38 Aquifer: FYROM – Central Serbia |  |  | - Shared by: FYROM and Serbia |  |
| --- | --- | --- | --- | --- |
| - Type 5, Mesozoic, Paleozoic, rocky masses (generally schists and limestones) with karstic aquifers, minor alluvial sediments, thickness of aquifers is variable depending on the tectonic characteristics, groundwater flows from Serbia to FYROM |  |  |  | Mediterranean Sea basin |
|  |  |  |  | Border length (km): 50 |
|  | - FYROM | - Serbia |  |  |
| - Area (km2) |  | - 100 |  |  |
| - Water uses and functions | - Water for drinking, little irrigation and mining | - More than 50% surface water |  |  |
| - Pressure factors | - Local leakage of groundwater from wells and groundwater from springs | - Waste disposal and industry |  |  |
| - Problems related to groundwater quantity | - Local reduction of boreholes yields and spring discharges | - Local reduction of baseflow in summer periods |  |  |
| - Problems related to groundwater quality | - Local from agriculture | - Moderate pollution drawn in to the aquifer system |  |  |
| - Transboundary impacts | - Only for quantity of groundwater | - None |  |  |
| - Groundwater management measures | - Monitoring of quantity and quality, protection zones, hydrogeological mapping, good agricultural practice, exchange of data between countries are used, other measures need to be applied or are planned. | - No management measures in place |  |  |
| - Status and what is most needed | - ? | - ? |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes | - After definition of the border between the two counties, all data must be defined and data must be exchanged |  |  |  |

| - INWEB no: 39 Aquifer: Tetovo - Gostivar |  |  | - Shared by: Serbia and FYROM |  |
| --- | --- | --- | --- | --- |
| - Type 2 and 5, Paleozoic rocky masses (generally schists) with karstic aquifers, minor alluvial sediments, thickness of aquifers is variable from 100 to 300 m, depending on the tectonic characteristics, groundwater flow occurs in both directions, but more from Serbia to FYROM |  |  |  | - Mediterranean Sea basin |
|  |  |  |  | - Border length (km): 15 |
|  | - Serbia | - FYROM |  |  |
| - Area (km2) | - 50 |  |  |  |
| - Water uses and functions | - 50% drinking water, 50% support of ecosystems | - Drinking water, irrigation, mining and industry |  |  |
| - Pressure factors | - Agriculture | - Local leakage of groundwater from wells and groundwater from springs |  |  |
| - Problems related to groundwater quantity | - None | - Reduction of groundwater level in boreholes and discharges of springs |  |  |
| - Problems related to groundwater quality | - Local only from farming | - None for quality |  |  |
| - Transboundary impacts | - None | - Only for quantity of groundwater |  |  |
| - Groundwater management measures | - No management measures introduced |  |  |  |
| - Status and what is most needed | - ? | - ? |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  | - After definition of the border between the two countries, all data must be defined and data must be exchanged |  |  |

| - INWEB no: 40 Aquifer: Korab/Bistra - Stogovo |  | - Shared by: Albania and FYROM |  |
| --- | --- | --- | --- |
| - Type 2 and 5 Mesozoic and Paleozoic schists and flysch sediments, containing Triassic evaporites (anhydrite and gypsum) and Triassic and Jurassic karstic limestones. Minor alluvial sediments with free (unconfined) groundwater, total aquifer thickness from 500 to 700 m, maximum 2000 m, strong links to surface waters, groundwater flow occurs in both directions, but more from FYROM to Albania - Groundwater provides >90% of total supply in Albania and FYROM |  |  | - Mediterranean Sea basin |
|  |  |  | - Border length (km): 25 |
|  | - Albania | - FYROM |  |
| - Area (km2) | - About 140 | - ? |  |
| - Water uses and functions | - 25-50 % for thermal spa, < 25 % each for drinking, irrigation and livestock | - Drinking water, irrigation, mining |  |
| - Pressure factors | - Waste disposal, sanitation and sewer leakage | - Groundwater abstraction, agriculture |  |
| - Problems related to groundwater quantity | - No problems | - Local reduction of discharge from springs |  |
| - Problems related to groundwater quality | - Pathogens from waste disposal, sanitation and sewer leakage | - None for quality |  |
| - Transboundary impacts | - None for quality and quantity | - Only for quantity |  |
| - Groundwater management measures | - Detailed hydrogeological mapping and vulnerability mapping, public awareness campaigns, delineation of protection zones and wastewater treatment are all needed. To increase the collaboration, to build up transboundary institutions and to create a joint programme for quantity and quality monitoring of the sulphur thermo-mineral springs issuing in both countries. | - Quantity and quality monitoring need to be improved, protection zones and all water activities, transboundary agreements and data exchange used, but need improvement |  |
| - Status and what is most needed | Not at risk at the moment Intensification of use of sulphur thermo-mineral groundwater by deep boreholes | - ? |  |
| - Future trends and prospects | Delineation the protection zones of the sulphur thermo-mineral springs and to improve the capture structures. |  |  |
| - Notes | Comparative study of the thermo-mineral springs of Albania and FYROM is needed. There are large fresh water karst springs issuing at high elevations |  |  |

| - INWEB no: 41 Aquifer: Jablanica/Golobordo |  | - Shared by: Albania and FYROM |  |  |
| --- | --- | --- | --- | --- |
| - Type 2, karstic limestones of average thickness 700 m and maximum 1500 m, weak links to surface waters, groundwater flow occurs in both directions - Groundwater is 70-80% of total water use in Albania |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 50 |
|  | - Albania |  | - FYROM |  |
| - Area (km2) | - 250 |  | - ? |  |
| - Water uses and functions | - 25-50% for irrigation, <25% each for drinking water and industry |  | - Drinking water supply, thermal water and industry, also HEP |  |
| - Pressure factors | - Modest pressures from waste disposal, sanitation and sewer leakage |  | - Sanitation and sewer leakage |  |
| - Problems related to groundwater quantity | - Local and moderate polluted water drawn into aquifer |  | - Local reduction of groundwater yields from wells and discharges from springs |  |
| - Problems related to groundwater quality | - Local and moderate pathogens from waste disposal |  | - None mentioned |  |
| - Transboundary impacts | - None for quantity or quality |  | - None for quantity and quality |  |
| - Groundwater management measures | - No management measures in place, many need to be introduced, detailed hydrogeological and vulnerability mapping, groundwater monitoring, public awareness, delineation of protection zones and wastewater treatment are all needed |  | - Monitoring of quantity and quality, protection zones, hydrogeological mapping, good agricultural practices, exchange of data between countries, other measures, need to be applied or are planned |  |
| - Trends and future prospects | The use of a large karst spring for the production of the electricity by Electric Power Station is planned |  |  |  |
| - Status and what is most needed | Not at risk at the moment, the population is small and the industry is not developed |  | - ? |  |
| - Notes | Surface karst phenomena are very well developed on Klenja plateau |  |  |  |

| - INWEB no: 45 Aquifer: Prespes and Ohrid Lakes |  |  | - Shared by: Albania, FYROM and Greece |  |
| --- | --- | --- | --- | --- |
| - Type 5, Mainly Triassic and Jurassic massive limestones, about 400 m thick and up to 550 m maximum, including Galicica mountain between the lakes, medium to strong links to surface water systems, groundwater flow dominantly from the basin of the Small Prespa Lake to the Big Prespa Lake and from there to the Ohrid lake basin. Groundwater movement is interconnected between all three countries. - Groundwater provides greater than 80% of total water use |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 40 (GR/AL), 20 (GR/FYROM) |
|  | - Albania | - FYROM |  | - Greece |
| - Area (km2) | - 750 | - ? |  | - 413 |
| - Water uses and functions | - 25-50% for irrigation and <25% each for drinking water and industry industry, support for baseflow and ecosystems | - Drinking water, industry and ecosystems |  | - Irrigation, water supply (no figures given) and support of ecosystems |
| - Pressure factors | - Minor sanitation and sewer leakage | - Minor sanitation |  | - ? |
| - Problems related to groundwater quantity | - Widespread but moderate degradation of ecosystems, and polluted water drawn into aquifer | - Local and moderate reduction of groundwater level, yields of wells and discharges of springs |  | - None significant |
| - Problems related to groundwater quality | - Local and moderate nitrogen and pathogens in both groundwater and lakes, but the trend is increasing | - None mentioned |  | - None significant |
| - Transboundary impacts | - A slight increase in the phosphorus in Lake Ohrid | - None mentioned |  | - ? |
| - Groundwater management measures | - No management measures in place, many need to be introduced:- transboundary institutions, water use efficiency, monitoring of groundwater and lakes, protection zones, vulnerability mapping, priorities for wastewater treatment, integration with Prespa and Ohrid lakes basin management | - Monitoring of groundwater, must be improved with agreements, data exchange, hydrogeological databases, planned together |  | - Abstraction management, monitoring, public awareness and data exchange need improvement, other measures need introducing or are planned |
| - Trends and future prospects | - Increasing groundwater use by the growing population and intensive development of tourism. Increasing collaboration of all three countries to protect groundwater and surface water resources in a basin-wide way |  |  |  |
| - Status and what is most needed | - Small risk at the moment. Increasing risk of contamination of karst water and of the lakes in the future by the increasing population and tourism | - ? |  | - ? |
| - Notes | - Ohrid lake is intensively recharged from Prespa Lake through the Mali Thate-Galicica karst massive. Large karst springs with average discharge about 10 m3/s issue near the Albanian-FYROM border at the edge of Lake Ohrid |  |  | - Monitoring is important in relation to the ecosystems supported by Prespa Lake |

| - INWEB no: 44 Aquifer: Vjosa-Pogoni/Nemechka |  |  | - Shared by: Albania and Greece |  |
| --- | --- | --- | --- | --- |
| - Type 1, Succession of large anticlines containing karstic limestones of mainly Jurassic and Cretaceous age and synclines with formations of Palaeocene and Eocene flysch; average thickness about 2500 m, maximum more than 4000 m, the complicated geological structures and hydrogeological conditions which bring these formations together produce large karst springs, groundwater discharges towards both countries, weak links to surface waters. - Groundwater provides more than 90% of total water use |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 37 |
|  | - Greece | - Albania |  |  |
| - Area (km2) | - 350 | - 550 |  |  |
| - Water uses and functions | - Water supply with minor irrigation and fish farming; support of ecosystems | - 25-50% irrigation, <25% each for drinking water, livestock and industry, maintaining baseflow and springs and supporting ecosystems |  |  |
| - Pressure factors | - Minimal due to very small population | - Minor waste disposal and sewer leakage |  |  |
| - Problems related to groundwater quantity | - None | - Local and moderate degradation of ecosystems |  |  |
| - Problems related to groundwater quality | - Some high natural concentrations of metals | - Local and moderate pathogens |  |  |
| - Transboundary impacts | - None for quantity or quality | - None for quantity or quality |  |  |
| - Groundwater management measures | - Existing abstraction control and monitoring need improvement, other measures need to be applied or are planned | - None already used, but a range of measures need to be applied, detailed hydrogeological and vulnerability mapping, groundwater monitoring, public awareness, delineation of protection zones and wastewater treatment |  |  |
| - Trends and future prospects |  |  |  |  |
| - Status and what is most needed | - ? | - No risk at the moment, the population is small and industry is not developed |  |  |
| - Notes | - Large spring discharges of Kalama, Gormou and Drinou | - Large karst groundwater quantities (average about 8 m3/s) discharge in the Vjosa River gorge in Albanian territory. There are also other large karst springs, the Glina sulphate spring is a well known bottled karst spring |  |  |

| - INWEB no: 43 Aquifer: Mourgana Mountain/Mali Gjere |  |  | - Shared by: Greece and Albania |  |
| --- | --- | --- | --- | --- |
| - Type 5, karstic aquifer developed in Jurassic and Cretaceous limestones in large anticlines with flysch in synclines. Average thickness about 3000 m, maximum >4000 m. Thickness of alluvial gravels in the Drinos River 20-80 m. Strong links to surface water systems. Little groundwater flow across the border. The Drinos River flowing from Greece to Albania recharges the alluvial aquifer which contributes to the Bistritsa (Blue Eye) Spring (average discharge 18.5 m3/s) in Albania. The Lista Spring (average 1.5 m3/s) issues in Greece. - Groundwater provides about 90% of total water use |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 20 |
|  | - Greece | - Albania |  |  |
| - Area (km2) | - 200? | - 440 |  |  |
| - Water uses and functions | - No major water uses due to small population, some local livestock, minor irrigation and fish farming from springs | - Provides 100% of drinking water supply and spa use, and >75% for irrigation, industry and livestock |  |  |
| - Pressure factors | - None, low population in mountain area | - Minor from waste disposal and sewer leakage |  |  |
| - Problems related to groundwater quantity | - None | - Some local and moderate drawing of polluted water into the aquifer. No declines in groundwater level |  |  |
| - Problems related to groundwater quality | - None | - Widespread but moderate salinisation – the alluvial groundwater has high sulphate (300 -750 mg/l), which contributes to increased average sulphate (135 mg/l) in Blue Eye Spring |  |  |
| - Transboundary impacts | - Neither for quantity or quality | - None |  |  |
| - Groundwater management measures | - Existing abstraction control and monitoring need to be improved, other management measures are needed or planned | - No measures employed, those needed include detailed hydrogeological and groundwater vulnerability mapping, public awareness, delineation of protection zones and wastewater treatment. Also to increase collaboration, to build up transboundary institutions and to create a joint basin wide programme for quantity and quality monitoring |  |  |
| - Trends and future prospects |  | - Increased use of groundwater in alluvial deposits and………… |  |  |
| - Status and what is most needed | - ? | - Small risk at the moment, but with increasing tendency because the area is rapidly developing, both industrial and agricultural |  |  |
| - Notes |  | According to a preliminary proposal, about 4.5 m3/s of water from Blue Eye spring will be exported to Puglia - Italy through an undersea water supply pipeline. |  |  |

| - INWEB no: 47 Aquifer: Pelagonia/Florina/Bitolsko |  | - Shared by: Greece and FYROM |  |
| --- | --- | --- | --- |
| - Type 3/4, Quaternary and Neogene alluvial sands and gravels with some clay and silt and cobbles, up to 300 m thick, overlying Palaeozoic and Mesozoic schists |  |  | - Mediterranean Sea Basin |
|  |  |  | - Border length (km): |
|  | - Greece | - FYROM |  |
| - Area (km2) | - 607 | - ? |  |
| - Water uses and functions | - Water supply and irrigation | - Drinking water supply, support of ecosystems and agriculture and maintaining baseflow and springs |  |
| - Pressure factors | - Agriculture, coal mining | - Groundwater abstraction |  |
| - Problems related to groundwater quantity | - None | - Widespread and severe increase of pumping lifts, degradation of ecosystems and drawing of polluted water into aquifer, widespread but moderate reduction of borehole yields, local but severe reduction in baseflow and spring flow |  |
| - Problems related to groundwater quality | - Nitrate, heavy metals | Salinization, nitrogen, pesticides, heavy metals, pathogens, industrial organic compounds and hydrocarbons |  |
| - Transboundary impacts | - None | None for quantity or quality |  |
| - Groundwater management measures | - Existing abstraction controls, water use efficiency, monitoring and wastewater treatment need to be improved, other measures are mentioned as needed or currently planned | Increasing efficiency of groundwater use, monitoring of quantity and quality, public awareness, protection zones, vulnerability mapping, good agricultural practices, exchange of data between countries and treatment of industrial effluents need to be improved, other measures need to be applied or are planned |  |
| - Trends and future prospects |  |  |  |
| - Status and what is most needed | - ? | - ? |  |
| - Notes |  |  |  |

| - INWEB no: 48 Aquifer: Gevgelija |  | - Shared by: FYROM and Greece |  |  |
| --- | --- | --- | --- | --- |
| Type 1 or 5, Quaternary alluvial sediments, sands with gravel, partly clayey and silty with cobbles of bedrock - diabases, biotite gneisses and shists. Average thickness of 10 m and maximum 100 m. Medium link with surface water systems, groundwater flow from FYROM to Greece. |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): |
|  | - FYROM |  | - Greece |  |
| - Area (km2) | - ? |  | - ? |  |
| - Water uses and functions | - Maintaining baseflow and springs and support of ecosystems |  |  |  |
| - Pressure factors | - Abstraction of groundwater, agriculture |  |  |  |
| - Problems related to groundwater quantity | - Extensive and severe increases in pumping lifts, reduction in borehole yields, degradation of ecosystems and drawing in of polluted water, local and severe reduction of baseflow and springflow |  |  |  |
| - Problems related to groundwater quality | Salinization of natural origins and Nitrogen, pesticides, heavy metals, pathogens, industrial organics and hydrocarbons |  |  |  |
| - Transboundary impacts | Observed both decline of groundwater levels and increased groundwater pollution |  |  |  |
| - Groundwater management measures | Existing efficiency of groundwater use, monitoring of quantity and quality, public awareness, protection zones, vulnerability mapping, agricultural practice, data exchange and treatment need improvement, other measures need to be applied or are planned |  |  |  |
| - Status and what is needed | - ? |  | - ? |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 49 Aquifer: DOJRAN LAKE |  | - Shared by: Greece and FYROM |  |
| --- | --- | --- | --- |
| Type 3, Quaternary and Upper Eocene alluvial aquifer overlying metamorphic rocks, sedimentary sequences and carbonate formations -Precambrian, older Paleozoic and Green Metamorphic Complex. Strong links with surface water systems, groundwater flow is generally towards the lake. The catchment of the Lake covers a total of 280 km2 |  |  | - Mediterranean Sea basin |
|  |  |  | - Border length (km): |
|  | - Greece | - FYROM |  |
| - Area (km2) | 190 | - 92 |  |
| - Water uses and functions | - Maintaining baseflow and springs and support of ecosystems and support of agriculture –irrigation | - Irrigation and water supply |  |
| - Pressure factors | Groundwater abstraction for irrigation | - ? |  |
| - Problems related to groundwater quantity | Widespread but moderate reduction in borehole yields, baseflow and degradation of ecosystems, the lake volume and area has declined drastically | - Declining groundwater levels, reduction of water from the lake, degradation of associated ecosystems |  |
| - Problems related to groundwater quality | Heavy metals | - None |  |
| - Transboundary impacts | ? | - For quantity only |  |
| - Groundwater management measures | Existing groundwater abstraction control, monitoring of groundwater quantity and quality, data exhange and wastewater treatment need to be improved, other management measures are needed or currently planned | - Existing efficiency of groundwater and lake water use, monitoring of quantity and quality of the lake, level of the lake, wells on both sides, public awareness, protection zones, vulnerability mapping, data exchange and treatment need improvement or are planned measures. |  |
| - Status and what is needed | - ? | - Serious decline in lake level and area, groundwater abstraction to help recover lake levels has been tried |  |
| - Trends and future prospects |  |  |  |
| - Notes |  |  |  |

| - INWEB no: 50 Aquifer: Sandansky - Petrich |  |  | - Shared by: Bulgaria, Greece and FYROM |  |  |
| --- | --- | --- | --- | --- | --- |
| - Type 5, Alluvial sediments with aquifer with free level of ground water from 10 to 100 m, thermal water is characterizated from 100 to 300 m, Paleozoic rocky masses with schists and Paleozoic limestones with karst aquifers with different quantity of ground water, flow occurs in both directions but more from FYROM to Bulgaria and Greece |  |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  |  | - Border length (km): - BG/GR – 18, BG/FYROM 5 |
|  | - Bulgaria | - Greece |  | - FYROM |  |
| - Area (km2) | - 764? | - ? |  | - ? |  |
| - Water uses and functions |  |  |  | - Drinking water, irrigation and industry, thermal springs, agriculture |  |
| - Pressure factors |  |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  | - None mentioned |  |
| - Problems related to groundwater quality |  |  |  |  |  |
| - Transboundary impacts |  |  |  |  |  |
| - Groundwater management measures |  |  |  | - Protection zones need to be improved, monitoring systems, exchange of data and other measures need to be introduced |  |
| - Status and what is needed | - ? | - ? |  | - ? |  |
| - Future trends and prospects |  |  |  |  |  |
| - Notes |  |  |  |  |  |

| - INWEB no: 51 Aquifer: GOTZE DELCHEV/ ORVILOS-AGISTROS |  |  | - Shared by: Greece and Bulgaria |  |
| --- | --- | --- | --- | --- |
| Type? Karstic aquifer formed in the Proterozoic crystalline schist of the Rodopi with thick marbles overlying gneiss, some Pleistocene alluvial sediments at the edges. Groundwater levels at 650 m in the east part of the system and at 305 m in the west part show there is more than one aquifer with unknown hydrogeological relation between them which is under further investigation |  |  |  | - Mediterranean Sea Basin |
|  |  |  |  | - Border length (km): 22 |
|  | - Greece | - Bulgaria |  |  |
| - Area (km2) | - 200 |  |  |  |
| - Water uses and functions | - No direct uses |  |  |  |
| - Pressure factors | - None mentioned |  |  |  |
| - Problems related to groundwater quantity | - None |  |  |  |
| - Problems related to groundwater quality | - None |  |  |  |
| - Transboundary impacts | None |  |  |  |
| - Groundwater management measures | Existing groundwater abstraction regulation and monitoring of groundwater quantity need to be improved, other management measures are needed or planned |  |  |  |
| - Status and what is needed | - ? |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 52 Aquifer: Nastan- Trigrad |  | - Shared by: Greece and Bulgaria |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece |  | - Bulgaria |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 53 Aquifer: Smolyan |  | - Shared by: Greece and Bulgaria |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece |  | - Bulgaria |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 54 Aquifer: Rudozem |  | - Shared by: Greece and Bulgaria |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece |  | - Bulgaria |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 55 Aquifer: Erma Reka |  | - Shared by: Greece and Bulgaria |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece |  | - Bulgaria |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Future trends and prospects |  |  |  |  |
| - Notes |  |  |  |  |

| INWEB no: 56 Aquifer: SVILENGRAD STAMBOLO/ORESTIADA |  |  | - Shared by: Greece, Bulgaria and Turkey |  |
| --- | --- | --- | --- | --- |
| - Type 4, Quaternary and Upper Neogene lake and river alluvial sands, clayey sands, gravels, sandy clays and clays overlying the metamorphic rocks of the Rodopi Massif. Strong links with surface water systems and discharge towards the rivers Ardas and Evros. |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. or Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece | - Bulgaria |  |  |
| - Area (km2) | 600 | - 150? |  |  |
| - Water uses and functions | - 90% for irrigation and 10% - for drinking water supply, maintaining baseflow and springs and support of ecosystems |  |  |  |
| - Pressure factors | - Agriculture |  |  |  |
| - Problems related to groundwater quantity | - Although abstraction is reported to greatly exceed recharge, no problems were mentioned |  |  |  |
| - Problems related to groundwater quality | Nitrogen and pesticides from agriculture |  |  |  |
| - Transboundary impacts | Observed decline in groundwater levels and pollution |  |  |  |
| - Groundwater management measures | Existing groundwater abstraction regulation, monitoring of groundwater quantity and quality and effluent reuse and treatment need to be improved, other measures need to be applied or are planned |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 57 Aquifer: Evros/Meric |  | - Shared by: Greece and Turkey |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Greece |  | - Turkey |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Notes |  |  |  |  |
| - Future trends and prospects |  |  |  |  |

| - INWEB no: 58 Aquifer: Topolovgrad Massif |  |  | - Shared by: Bulgaria and Turkey |  |
| --- | --- | --- | --- | --- |
| - Type 2, Proterozoic, Paleozoic, Triassic, Jurassickarstic limestones, marbles, sands, sandstones, clay materials, effusion and intrusion rocks, medium links with surface water systems: Dominant groundwater flow direction: from W-SW to E-NE towards Turkey - Proportion groundwater of total use is not known |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Bulgaria | - Turkey |  |  |
| - Area (km2) | - 4,450 |  |  |  |
| - Water uses and functions | - 25 – 50 % Drinking water supply, < 25 % each for irrigation and livestock Maintaining baseflow and springs and - support of ecosystems and of agriculture |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | Nitrate in NE part |  |  |  |
| - Transboundary impacts | None for quantity or quality |  |  |  |
| - Groundwater management measures | Existing groundwater abstraction by regulation needs to be improved, several other measures mentioned as needing to be applied or currently planned, including monitoring of quality and quantity |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Trends and future prospects | - ? |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 59 Aquifer: Malko Tarnovo Massif |  | - Shared by: Bulgaria and Turkey |  |
| --- | --- | --- | --- |
|  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |
|  | - Bulgaria | - Turkey |  |
| - Area (km2) |  |  |  |
| - Water uses and functions |  |  |  |
| - Pressure factors |  |  |  |
| - Problems related to groundwater quantity |  |  |  |
| - Problems related to groundwater quality |  |  |  |
| - Transboundary impacts |  |  |  |
| - Groundwater management measures |  |  |  |
| - Status and what is needed |  |  |  |
| - Future trends and prospects |  |  |  |
| - Notes |  |  |  |

| INWEB no: 60 Aquifer: Pleistocene Someş/Szamos alluvial fan |  |  | - Shared by: Romania and Hungary |  |
| --- | --- | --- | --- | --- |
| Type 4, Holocene-Lower Pleistocene alluvial sediments of sands, clayey sands, gravels and even boulders, weak to medium links with surface water systems. In Romania, the shallow (15 -30 m) Holocene unconfined upper part (ROSO01) and the confined Lower Pleistocene (ROSO13), varying from 40 m thick in the west to 130 m are considered separate groundwater bodies. Mean thickness 180 m and maximum 470 m in the Hungarian part. Dominant groundwater flow from East (Romania). to West (Hungary). More than 80% of total water use is from groundwater in the Hungarian part. |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 64 |
|  | - Romania | - Hungary |  |  |
| - Area (km2) | - 1,380 | - 976 |  |  |
| - Water uses and functions | - Upper, 40% industry, 30% each irrigation and drinking water; lower, 75% for drinking water supply and 25 % for industry, minor agricultural use | - >75% drinking water supply, less than 10% each for irrigation, industry and livestock, maintaining baseflow and support of ecosystems |  |  |
| - Pressure factors | - ? | - Agriculture, sewers and septic tanks |  |  |
| - Problems related to groundwater quantity | - Local and moderate increased pumping lifts and small drawdowns only around two major catchments | - Local and moderate increases in pumping lifts, reduction in borehole yield, reduced spring flow and degradation of ecosystems |  |  |
| - Problems related to groundwater quality | - None mentioned | Widespread but moderate nitrate, up to 200 mg/l, local and moderate pesticides up to 0.1 g/l and widespread but moderate arsenic at up to 50 g/l |  |  |
| - Transboundary impacts | None for quantity or quality | - None |  |  |
| - Groundwater management measures | Vulnerability mapping for land use planning needs to be applied, and a range of other measures are currently planned | - Groundwater abstraction control by regulation effective, control by financial mechanisms, water use efficiency, monitoring, public awareness, protection zones, wastewater treatment, data exchange and arsenic removal all need improvement, vulnerability mapping and improved agricultural practices, integration into river basin management are needed |  |  |
| - Status and what is needed | - ? | - ? - Evaluation of the utilisable resources, quality status, |  |  |
| - Notes |  |  |  |  |
| - Trends and future prospects |  | - Joint monitoring (mainly quantitative) is needed and the existing joint modelling should be updated |  |  |

| - INWEB no: 61 Aquifer: Pleistocene Mureş/Maros alluvial fan |  |  | - Shared by: Romania and Hungary |  |
| --- | --- | --- | --- | --- |
| - Type 4, Pleistocene and Holocene alluvial sediments, predominantly pebbles, sands and silts, weak to medium links with surface water systems, mean thickness 200 m and maximum 500 m, groundwater flow from SE (Romania to NW (Hungary). In Romania the shallow (15-30 m) upper part is considered to be a separate aquifer (ROMU 20) than the deeper, confined part of the sequence (ROMU22). Groundwater is 80% of total use use in Hungary. |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): |
|  | - Romania | - Hungary |  |  |
| - Area (km2) | - 2200 | - 4319 |  |  |
| - Water uses and functions | - 75 % for drinking water supply, 15% for industry and 10 % for irrigation (shallow), 45%, 35% and 20% respoectively for the confined aquifer | - >75% drinking water, <25% each for irrigation, industry and livestock, support of agriculture and ecosystems |  |  |
| - Pressure factors | - Groundwater abstraction | - Groundwater abstraction, agriculture, septic tanks |  |  |
| - Problems related to groundwater quantity | - Local and moderate increased pumping lifts and local small drawdowns only around two important catchments | - Local and moderate increase in pumping lifts, reduction in yields and reduced baseflow, local but severe degradation of ecosystems |  |  |
| - Problems related to groundwater quality | - None mentioned | - Widespread but moderate nitrate at up to 200 mg/l, local and moderate pesticides at up to 0.1 µg/l, widespread and sever arsenic at up to 300 µg/l |  |  |
| - Transboundary impacts | None for quantity or quality | - No |  |  |
| - Groundwater management measures | Vulnerability mapping for land use planning needs to be applied , range of other measures currently planned | - Groundwater abstraction by regulation is already used and effective, transboundary agreements, improved efficiency, monitoring, public awareness, protection zones and wastewater treatment and arsenic removal need improvement, vulnerability mapping, good agricultural practices and priorities for wastewater treatment, integration with river basin management need to be applied |  |  |
| - Status and what is needed | - ? | - Evaluation of the utilisable resources, quality status, joint monitoring (mainly quantitative) and joint modelling is needed |  |  |
| - Notes |  |  |  |  |
| - Trends and future prospects |  | - Water importation because of arsenic may be required |  |  |

| - INWEB no: 63 Aquifer: Middle Sarmantian Pontian |  |  | - Shared by: Romania and Moldova |  |
| --- | --- | --- | --- | --- |
| - Type 4, Middle Sarmatian – Pontian sediments from the Central Moldovian Plateau, predominantly sands, sandstones and limestones, weak links with surface water systems, dominant groundwater flow direction: from.East (Romania) to West (Moldova) |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Romania | - Moldova |  |  |
| - Area (km2) | - 11,964 |  |  |  |
| - Water uses and functions | - 50 % dinking water supply, 25 % - industry and 15% irrigation, minor spa |  |  |  |
| - Pessure factors | - None mentioned |  |  |  |
| - Problems related to groundwater quantity | - None mentioned |  |  |  |
| - Problems related to groundwater quality | Local, moderate to severe salinity |  |  |  |
| Transboundary impacts | None for quantity or quality |  |  |  |
| - Groundwater management measures | Transboundary institutions already used and effective for this groundwater, other management measures need to be applied or are currently planned |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |

| - INWEB no: 64 Aquifer: Neogene-Sarmatian |  |  | - Shared by: Bulgaria and Romania |  |
| --- | --- | --- | --- | --- |
| - Type 1 or Type 4? Neogene – Sarmatian oolitic and organogenic limestones, sands, sandstones and clays up to 250 m thick, weak to medium links with surface water systems, dominant groundwater flow from W-SW (Bulgaria) to E-NE (Romania) |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 90 |
|  | - Bulgaria | - Romania |  |  |
| - Area (km2) | - 4450 | - 2,178 |  |  |
| - Water uses and functions | - 25 – 50 % for drinking water, < 25 % each for irrigation, industry and livestock, also maintaining baseflow and springs, support of ecosystems and agriculture | - 50 % drinking water supply, 30 % irrigation and 20% for industry |  |  |
| - Pressure factors | - Agriculture | - None mentioned |  |  |
| - Problems related to groundwater quantity | - Local and moderate reduction of borehole yields | - None mentioned |  |  |
| - Problems related to groundwater quality | Local and moderate concentrations (10 – 100 mg/l) of nitrogen from agricultural sources | - None reported |  |  |
| - Transboundary impacts | - None for quantity or quality | - No |  |  |
| - Groundwater management measures | - Control of abstraction used and effective, transboundary agreements, monitoring, protection zones, vulnerability mapping, effluent treatment used but need improvement, other measures needed or currently planned | - None reported as already in use, a range of measures are currently planned |  |  |
| - Status and what is needed | - ? | - ? |  |  |
| - Notes | - Groundwater provides about 30 % of total water use |  |  |  |
| - Trends and future prospects |  |  |  |  |

| - INWEB no: 65 Aquifer: Upper Jurassic – Lower Cretaceous |  |  | - Shared by: Bulgaria and Romania |  |
| --- | --- | --- | --- | --- |
| - Type 4, Upper Jurassic –Lower Cretaceous karstic limestones, dolomites and dolomitic limestones, weak links with surface water systems, groundwater flow from Bulgaria to Romania Groundwater is about 40% of total water use |  |  |  | - Black Sea Basin |
|  |  |  |  | - Border length (km): 290 |
|  | - Bulgaria | - Romania |  |  |
| - Area (km2) | 15, 476 | - 11,427 |  |  |
| - Water uses and functions | - 25-50% for drinking water supply, <25 % for irrigation | - 70 % for drinking water supply, 15% each for irrigation and industry |  |  |
| - Pressure factors | - Agriculture |  |  |  |
| - Problems related to groundwater quantity | - Local but severe increased pumping lifts | - Local and moderate increased pumping lifts |  |  |
| - Problems related to groundwater quality | Local and moderate concentrations (30 – 60 mg/l) of nitrogen species from agriculture | - None mentioned |  |  |
| - Transboundary impacts | None for quantity or quality | - None |  |  |
| - Groundwater management measures | Groundwater abstraction regulation already used and effective, transboundary institutions, monitoring of groundwater quantity and quality, protection zones, vulnerability mapping, good agricultural practices and wastewater and effluent treatment used but need improvement, exchange of data is needed | - No management measures reported as being in use, a range of measures is currently planned |  |  |
| - Status and what is needed | - ? | - ? |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |

`

| - INWEB no: XX Aquifer: Pester |  |  | - Shared by: Serbia and Montenegro |  |
| --- | --- | --- | --- | --- |
| - Type2, Middle Triassic karstic limestones up to 1000 m thick, weak links to surface water systems, dominant groundwater flow is towards the south west - Groundwater provides 80% of total water use |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
|  | - Serbia | - Montenegro |  |  |
| - Area (km2) | - 407 / >1,500 |  |  |  |
| - Water uses and functions | - Irrelevant groundwater resource (no demand for groundwater ) |  |  |  |
| - Pressure factors | - <25% for drinking water, industry - and mining |  |  |  |
| - Problems related to groundwater quantity | - None reported |  |  |  |
| - Problems related to groundwater quality | - None reported |  |  |  |
| - Transboundary impacts | None |  |  |  |
| - Groundwater management measures | None reported as being in use, a whole range of measures mentioned as needing to be applied, including monitoring of quantity and quality |  |  |  |
| - Notes | - 2 questionnaires differ on water uses |  |  |  |
| - Trends and future prospects |  |  |  |  |
| - Notes |  |  |  |  |
| - Status and what is needed | - ? |  |  |  |

| - INWEB no: Aquifer: |  | - Shared by: |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Border length (km): |
| It belongs to the [Black Sea basin] [Mediterranean Sea basin]. Or - Due to the complicated hydrogeological conditions and still missing data it cannot yet be attached to the Black Sea or Mediterranean Sea basins. |  |  |  |  |
| - Area (km2) |  |  |  |  |
| - Water uses and functions |  |  |  |  |
| - Pressure factors |  |  |  |  |
| - Problems related to groundwater quantity |  |  |  |  |
| - Problems related to groundwater quality |  |  |  |  |
| - Transboundary impacts |  |  |  |  |
| - Groundwater management measures |  |  |  |  |
| - Notes: |  |  |  |  |
| - Status and what is needed |  |  |  |  |
| - Future trends and prospects |  |  |  |  |